1

Quick Start Guide

1 Assembly

Notices:

. The power supply range of MC (Autopilot) and WIFI is 3s~6s(Power remains a

little), namely 10.8V~25.5V, MC provides 5.7V power to RC receiver

automatically, no need to add any out-built power module. After correct

connection and powering it, the autopilot will complete initialization in few

seconds and LED light starts to blink in red for three times continuously,

indicating the connection is correct and motivated successfully.

. The above "the light blinks in red for three times" is a sign to check whether all

hardware are working normally or not, all data connection is based on this. After

powering if no "the light blinks in red for three times", please contact Zero UAV

(Beijing) Intelligence Technology Co. Ltd. or its distributors directly. Otherwise,

please check the assembly carefully referring to the above illustration.

2

Attention:

Any wrong connection of IMU or GPS may result in burning out the devicesI

. When ESC and motors working,they will produce serious magnetic

interference, so GPS and magnetic sensor module must be installed by

nonferrous to keep far away from motor and ESC (especially the Octo-rotor),

and the arrow is towards aircraft nose. Otherwise, the aircraft will fly circling

around and incorrectly hover. As there are rich eletric current in the batterywire

which can produce magnetic field, users must place the batterywire far away

from GPS and magnetic sensor module, otherwise the craft will fly in circle in

GPS mode.

. When using gimbal, user need to power on the servo. Just leave one ESC

powered on if ESCs come with bec function and dump othersf power. Plug the

battery of one servo to any of the M1~M8 if ESCs come without bec function.

Warning:

When using image transmission, users must make the image transmission far

away from IMU to avoid the interference.

3

2 Debugging and WIFI Set

STEP1: Download YS-GCS.

It must be installed to the Mobile memory card. Android system can be installed

automatically via running file installation once in the file manager. Apple system

can be installed after APP jailbreak, please refer toh Apple Installation

Instructionsh

STEP2: WIFI Connection.

Tips:

If the serial number of YS-X6 isFYS-X6-Serial No. N(Example: YS-X6-10112N)C

It indicates that the WIFI module is integrated with router function and works in

point-to .point.If the serial number has no gNh, then the WIFI module is

ordinary without router function.

. Serial number of YS-X6 with gNh (work in point-to-point ), shown as

below:

Communication: Wireless router is no need. After WIFI works, your Phone or

Tablet will search for a wireless network whose name is YS-X6-Serial No.N

(Example: YS-X6-10112N), then please put in the password:12345 and connect

it. If the communication is successful, data will be displayed when GCS is

4

enabled. Please refer to the appendix4 to configurate the Wifi to be ordinary if

your phone canft be compatible with Wifi module whose serial number comes

with gNh.

. Serial number of YS-X6 without gNh, shown as below:

Communication: Wireless router is needed to build AP. Set SSID of the router

ashYS-X6-Serial No.h (Example: YS-X6-10112N) (Note: The letters must be in

Capital), password is 82890430, use WPA2-PSK AES as encryption method and

set Router IP as 192.168.1.1 (Phone with hot spot also can be used instead of

router, but must make sure SSID and password can be set up on the phone, in

same way IP should be: 192.168.1.1) The indicator light on WIFI will be going

on in blue if the WIFI module successfully connected to AP.

Then enable the WLAN on Phone/Tablet to connect to router.

Tips:

. All the flight data can be saved to the files as following after WIFI data

connected.

To Android: the file with name of gT+runtimeh saved in folder gYshjh under root

directory of Phone/TabletG

TO Apple (IOS 5.0/5.0.1/5.1/5.1.1 ): the folder:

/private/var/mobile/documents/yswifi

. The file can be used for data analysis through the Replay Tool on PC.

5

3. Installation Guide

After connecting all data, you can set up the below options as showing in

g Installation Guideh of GCS gSettingsh (If the data is not connected, please read

the above two steps carefully).

Please refer to the diagram as below:

(1)Channel Calibration.

(2)Check CH5 & CH6, set up F/S (Fails safe).

(3)ESC stroke calibration (This step is no need for special ESCs such as XA).

(4)Set up aircraft type and parameters. Fill in local magnetic declination (to

West is positive, to East is negative).

(5)Check the configuration direction of IMU & GPS.

After completing the installation guide, can start to fly manually to have a test.

6

Tips:

. About magnetic declination

Please refer to the below website for magnetic declination

http:// magnetic-declination.com/

http://www.ngdc.noaa.gov/geomagmodels/struts/calcDeclination

. About RC transmitterF

The RC transmitter must be set to fixed-wing mode and do not set any

mix-control. Any channel canft be inverted on FUTABA RC transmitter, but all of

the channels on Jr and WFLY RC transmitters need to be inverted.

For the RC transmitters of other branch, after channel calibration, check

whether the actual servo operation is the same as gManual servo positionh

showing on GCS , orientation/aileron/elevator display as left/right and push/pull,

hands off RC transmitter control displys as Neutral position, the max servo

position is 40, example: the max left servo displays as left40; the mini throttle

displays as 7 and the max as 90. It's correct if the actual servo operation is the

same as gManual servo positionh showing on GCS and if not same need to reset

channel to be reverse on transmitter.

7

4 Magnetic Course Calibration

There is no need to calibrate the magnetic course in manual mode, and the

gCourse angleh in data page of GCS will keep displaying as 0. But in GPS mode ,

the magnetic course calibration must be finished before take-off. Ferromagnetic

substances placed on multi-rotor or around its working environment will affect

the reading of earth magnetic for digital compass, it also reduces the accuracy

of the multi-rotor control, or even reads incorrect heading. Calibration will

eliminate such influences, and ensure MC system performs well in a non-ideal

magnetic environment. The magnetic data will display once after calibration and

be saved to the autopilot.

Notice:

Only changed to GPS mode, the actual course angle will display in Course angle

in data page of GCS.

STEP1 Calibration in horizontal:

Enter Calibration by elect gHorizontal Alignmenth from gMagnetic Compassh in

gSettingh page of GCS and click OK to send the command to autopilot. After

confirmationCthe status bar in the middle will display "Success" meaning the

autopilot received the command of Horizontal Alignment, can start the

Calibration: Rotate your aircraft 2~3 laps along with the horizontal surface, ask

a helper to monitor the Attitude angle changing in Data and try best to keep

Pitch and Roll value smaller than 3 (you can also check the blinking light which

is connecting with autopilot, if the light is keeping on it means attitude is good,

if the light is off it means the attitude is too big). After completing 2-3 laps

horizontal rotation, then go to the next step;

STEP2 Calibration in vertical:

Rotate the craft nose vertically facing the ground, select

gVertical Alignmenth from "Magnetic Compass" in GCS and click "OK" to send

8

the command to autopilot. After it says"Success" meaning the autopilot

received the command of Vertical Alignment and then the roll and pitch

will change close to 0 degree slowly via checking the attitude angle in

gDatah(namely changed reference coordinate system, itfs level when

the craft head face the ground).Then hold your Multi-rotor vertically and

rotate it along with its vertical axis keeping the attitude angle smaller than 3,

after completing 2-3 laps vertical rotation,then go to the next step; you can

also check the blinking light which is connecting with autopilot, constantly

light on means attitude is good, if attitude is too big the light will turn off .

STEP3

After rotating, in settings page click on Magnetic Compass --- save Magnetic

Compass---OK. After you finish the calibration, the GCS will go to "Control"

interface automatically. After waiting for scores of seconds i the purple light

keeps going on during data readingj, mobile will display the magnetic sensor

data of AP, only look the red and blue circle in the middle of cross coordinates,

if they are close to standard round, then indicates a successful calibration and

data in good condition.

Two round circles will show whether the calibration was successful or not:

If two round circles show up in the "Control" interface (one blue circle one red

circle), calibration succeeds, can exit ;

If no circle or only one circle shows up in the "Control" interface, the calibration

has failed. Enter calibration by selecting " Horizontal Asignment" and then

re-start from step 1for re-calibration.

9

Successful Calibration Image:

TipsF

After correct assembly and successful magnetic compass calibration once, no

need to recalibrate without any constructional reconfiguration or only upgrade

the firmware.

10

5 Control Priority

Notices:

. User must select two 3-position switches and set them to CH5 & CH6 by RC

transmitter.

. Three modes can be switched through CH5, Manual/Manual Altitude Holding

/GPS. (Note:the flight status on gdatah page in GCS display gmanualh hmanualh

gauto-hoverh)

. CH6 can work only when CH5 stays at position 3 (namely GPS mode). CH6 is

for switching modes among Auto-hovering/ Auto-navigation /Auto-returning

home & landing.

. The switch of RC receiver is button which showing gReceiver offh or gReceiver

onh in GCS, used to turn off/on the RC receiver on craft ,namely any operation

is useless for aircraft if receiver displays gRC offh , which equals there is no RC

receiver on the craft.

It is strongly not suggested to close the receiver ( RC off) if user doesnft know

well about the system!

. Mobile RC Status means whether the cross operation is enabled or not in the

mobile RC interface, enable the GC software and click ong RC onh, there is a

ground circleh showing on the gcrossh ,indicating gMobile RC status hhas been

enabled g.Select g RC offh ,there is no round circle on the cross ,indicating

gMobile RC statush has been off.

11

RC

Status

CH5

Status

Mobile RC

Status

CH6

Status

Craft Status Priority

RC off X

Auto-Ena

ble X

When controlling by mobile, there is a

ground circleh showing on the gcrossh.

When losing WIFI or signal exceeding

10s, auto-returning home and landing.

(This mode is not commanded to

hobbyists.)

1

RC on

CH5

Manual X X

In full manual mode, itfs controlled by

RC transmitter only.

Note: No strong actions/ No high

speed when flying in manual mode.

2

CH5

Auto

(In

position

3j

Mobile

gControl

onh

CH6

position

1

When controlling by mobile, there is a

ground circleh showing on the gcrossh,

please check the mobile status, it has

nothing to do with the RC Transmitter.

3

CH6

position

2

Enabling waypoints on mobile can get

into gAuto-navigationh mode. (Make sure

the waypoints is uploaded)

CH6

position

3

gAuto-return home & landingh mode

Mobile

gControlh

off

CH6

position

1

gGPS auto-hoverh mode, if the

throttle is in the minimum, craft may go

down fast, meanwhile the throttle should

be maintained in middle-position.

4

CH6

position

2

Enabling waypoints on mobile can get

into gAuto-navigationh mode. (Make sure

the waypoints is uploaded)j

CH6

position

3

gAuto-return home & landingh mode

Remark:

. gXh: means irrelevant, any kind of status. When mobile remote control is

enabled, there is a round circle showing on the cross. When Mobile transmitter

is disabled, there is no round circle showing on the cross.

. gRC statush: means greceiver onh and greceiver offh status in GCSB

. gPriorityh: 1 means the highest.

12

6 Aircraft Type

To coaxial propellers: Blue propeller is at Top; Purple propeller is at Bottom.

Otherwise all propellers are at top. The arrow means the orientation aircraft

head should point to. Fill in gAircraft Typeh in the parameter settings with below

numbers.

13

Customized Parameters

Tips:

Customized parameters can be setup by customers to allocate motor

mix-control. And the proportion coefficient of pitch, toll and turning of each

motor also can be customized for applying to the irregular Multi-rotor frame.

. Customized throttle parameters: all of them must be 100.

. Customized course angle parameters: to make the aircraft rotate to right,

the changing method of corresponding motor speed should be decreased to

-100, and increase to 100.

. Customized pitch parameters: to make the aircraft head be downward, the

changing method of corresponding motor speed should be decreased to -100,

and increase to 100.

. Customized toll parametersFto make the aircraft roll to right, the changing

method of corresponding motor speed should be decreased to -100, and

increase to 100.

Now letfs take the Quad-X parameters setting for an example:

14

In GCS, please click on "Setting"--gParameters Defineh to fill in the parameters,

after verification change the parameter of aircraft type to 10.

Warning:

if the parameters are incorrectly setup, after powering autopilot on or the

"aircraft type" is changed to 10, the motors will immediately rotate with high

speed, so it's strongly suggested that take off all of the propellers before filling

in the defined parameters to ensure your absolute safety!)

And then click on "Setting"--gParameters Defineh--"Get" to verify the changing

is successful. Repower autopilot on after verification, push the throttle stick to

the minimum position to make the motor rotate in slow speed and meanwhile

carefully check whether the mix-control is consist with settings, after

verification can start to fly .

7 Arm/Disarm motors and Failsafe

Enable Motors Protection

Any time after landing or before take-off, only if it is in manual mode and the

throttle stick is at the bottom position, the motors can be locked up completely

after 5 seconds; After locked up, motors won't rotate even if push the throttle

stick. Only through genable insuranceh on phone or through motor arming

iplease refer to the tips of Motor Arming in part 9 Flightjthe motors can be

unlocked.

Disarm motor

Only in manual mode and without holding altitude, the motors stop rotating with

pulling the throttle stick back to the bottom. Other time, if pull back the throttle

stick to the bottom, the motors wonft stop rotating but only decrease altitude.

Attention:

During flying, if there is no accidents please do not switch to manual mode and

push the throttle to the minimum at the same time, otherwise the motors will

immediately stop rotating, craft will free-fall to the ground and never be

15

rebooted!

Tips:

Whenever need to stop the motors urgently, below ways can help:

(1) Switch CH5 to position 1, namely in manual mode but without holding

altitude, whatfs more the throttle stick must be in the minimum.

(2) when mobile selects "RC off", click on "motor off " on mobile RC interface; Or

enable the receiver "RC on", according to the method (1);

(3) After auto-returning home and landing on the ground, click on "motor off"

on mobile RC cross interface and confirm; Or enable the transmitter, according

to method (1);

(4) After landing on the ground in holding altitude or GPS mode, press the cross

downward continuously, namely the throttle stick or RC throttle stick controlling

landing is on the bottom position all the time, motor will stop rotating slowly till

shut down completely and locked up.

WIFI Signal Lost

(1) When receiver enabled (RC on), if the WIFI signal loses more than 1 second,

then control the aircraft by RC transmitter;

(2) When user select "receiver off " (RC off), all of the operations by transmitter

are unavailable, craft control is managed as follows:

A. In " waypoints flight " mode, the flight will not be influenced if the WIFI signal

loses, and aircraft will go back to the first waypoint to hover after finishing

waypoints flight, then you need to judge whether WIFI signal has recovered, if

the signal unrecoverable, then manage as following;

B. When itfs not g waypoints flight h mode or aircraft hovers in the first waypoint

after finishing waypoints flight, please wait 10 seconds if the WIFI signal loses,

and continue to operate if signal can recover; If there is no signal 10 seconds

later, then enable the returning & landing function; if the signal recovers again

16

during returning, then stop returning home instead of hovering. Continue to

return home and land if the signal is not recovered again.

RC(receiver) Signal Lost

Users must refer to the instruction of RC transmitter to correctly set up F/S,

set CH5 to position 3,CH6 to position 1, throttle stick to the middle position

(installation guide has tips). If enable F/S for any reason in gRC onh status, the

autopilot will be switched to auto- hovering status icontinue flight path for 5

seconds in auto-waypoints mode, wait for 5 seconds and start to return home if

the signal canft recover after 5 seconds.

17

8 Mobile Remote Control

(1) Phone Cross Control

During flight, when remote control is switched to gAuto-hoverh of g GPS modeh,

the craft shall get into auto-hover. Meanwhile click GCS on mobile¨ genable

control/disable controlh to enable or disable the control operation.

Mobile remote control operation ways as below:

RC ON

Transmitter Interface Operation Ways Craft Status

Circle area Red: Not loacated; Green: located. Keep still

Position Hold DO NOT touch any area Position hold.

Operation

1. Press the circle and move up/down

2. Press the circle and move left/right

3. Press the circle and move left top, left

bottom, right tip, etc.

1. Move back and front

2. Move left and right

3. Move

lefttop,leftbottom,etc

Climb/

Decline/Rota

te

Click the circle and move in the range of the

red cross:

1.Top

2.Bottom

3.Left

4.Right

Note

(1) The distance to the red dot means the

flight speed. The farther ,the faster.

(2) Press=moving,release=stop moving

(3) Release=position hold immediately

(4) The Maximum speed is 2.5 m/s

1AClimb

2ADecline

3ARotate to the left

4ARotate to the right

18

NoticeF

In gSettingsh of GCS , there is gchange altitudeh there, when in automatic mode

and the throttle stick is in the middle position, you can put in target altitude

directly (unit: meter) and verify, then the craft will go up or down to the preset

altitude.

i2jPhone attitude mode

User can enable phone attitude control mode in phone remote control mode,

click on genable attitudeh the transmitter interface will be changed to a blue

cross, which indicates mobile attitude control mode has been enabled.

Notice:

Keep the phone in horizon before enabling attitude mode, otherwise the craft

will lean to the direction where the phone tilt to.

Phone attitude operation is shown as the form below:

Enable

attitude

Transmitter Interface Operation Ways Craft Status

Circle

Status

Red:unlock

Green:lock

----

Hover

Operation

Keep the mobile phone flat and no click Hover

19

Notice:

The control mode can be switched between phone cross control and phone

attitude control.

Flight

Operation

1 Mobile phone tilt to front/back

2 Mobile phone tilt to left/right

3Mobile phone tilt to

upleft/downleft/upright/downright

1front/back

2left/right

3upleft/downleft/

upright/downr

ight move

Climb/Desc

end/Auto-ro

tate

1 click on right above the circle (inside the

cross)

2 click on under the circle (inside the cross)

3 click on the left of the circle (inside the

cross)

4 click on the right of the circle (inside the

cross)

Note:

(1)The further click point from the circle the

quicker craft moves

(2)Pressing time equals craft moving time.

1 Climb

2 Descend

3 Left rotation

4 Right rotation

20

9 Flight

i1j Motor Arming

Unlock throttle: Move rod as V shaped when the throttle is in the bottom. After

that push throttle stick to enable the motor in 5 seconds, and the motor

insurance will be locked up automatically exceeding 5 seconds.

Notices:

Rod moving: Rudder in the left- most position, elevator in the bottom, aileron in

the right-most position,throttle in the bottom. For the right or left throttle, user

need to judge by self that the motor arming direction is ”ªor V shape.

After moving rod as V shaped, motor will not be enabled automatically, user

need to push the throttle stick to the minimum position to enable the motor

after moving rod.

i2jVibrating and Shaking

During flight, user need observe the gvibrate stateh and gshake state h in gdatah

page to judge the shaking status of IMU. In stable flight, it is normal when the

gvibrate stateg and gshake state h range from 0~9, the smaller the number the

smaller the shaking, the number will directly affect flying performance.

Tips:

Vibrating coefficient:the maximum acceleration of alternate motion(vibrate) in

up&down/left&right/back&forth three directions.

Shaking coefficients:the maximum angular velocity of rotary movement around

X/Y/Z three axises.

(3) Manual servo position and real servo position

Manual servo position

21

After channel calibration on transmitter, when user put the stick in the middle

position, the value of rudder, elevator and aileron in gmanual servo positionh of

GCS should be small (no more than 2) . If user already adjusted the trim and the

manual servo position is not near the center, need to click on gNeutral Positionh

in gsettingsh to make the autopilot record the correct stick center position.

Real servo position

Real servo position is the relation of differential rotation speed outputted by

motor to keep aircraft stable during flight.

Tips:

User can judge the balance of motors and propellers by observing the real servo

position in level flying.

Example: when clockwise and anticlockwise propellers lose balance(suppose

clockwise motor is not leveling, the reaction torque generated by which is

smaller than that generated by anticlockwise motor), when in level flight and

heading unchangeable, the real rudder position will keep stable with left to 5~7.

With the same pulse width output, the reaction torque generated by clockwise

motor is smaller than that generated by anticlockwise in the same rotation

speed, which lead aircraft rotate to the right, autopilot will output levorotation

torque automatically, namely the differential make the clockwise rotation speed

faster and anticlockwise slower. In this condition, aircraft can fly normally and

do not need hover any more, but the power consumption and temperature of

clockwise motor are higher than the anticlockwise motorfs.

So, it is suggested that user need to precisely detect the balance and level

symmetry of motor and propeller to get a more stable flight. Until level flight

and hovering, the rudder of real servo position will be smaller (1~2) or around

neutral position.

(4) Auto-takeoff and Auto-landing

A.Semi-auto takeoff

When get 5 or more GPS satellites locks, switch CH5 to position 3 and CH6 to

22

position 1, that is the flight status is gauto-hoveringh, then push the throttle

stick after moving rod in V shaped, when throttle stick is more than half, aircraft

will take off automatically and auto-hover about 2~3 meters from the ground

and getting into auto-hovering status.

B. Full-auto takeoff

Notice:

Aircraft can take off fully-automatic only if the aircraft has completed

semi-automatic takeoff and the flying is successful.

Step 1 Wait until get 5 or more GPS satellites locked, put the THR stick in the

minimum position, switch CH5 to GPS mode (position 3 ), CH6 to position 1.

Step 2 Click on genable insuranceh, or move rods as V shaped to make the

throttle unlocked, then continue the next step within 5 seconds.

Step 3 Click on gauto-takeoff h, the aircraft will be powered on and take off

slowly, hovering in the height of 2~3 meters from ground.

Step 4 If the THR sticks is in the minimum position in auto-takeoff mode, if you

want to control the flying altitude, must move the THR to the neutral position.

Notice:

If there are any accidents during take-off you can switch to manual mode by

transmitter to control the craft .

C. Auto-returning home and Auto-landing

Autopilot will set the returning home position automatically when GPS has

locked or aircraft takes off in manual/auto mode.

In GPS mode, when switch CH6 to position 3 or select gauto-return home &

landingh in the mobile phone and meanwhile click on gOKh,AP will enable

auto-returning home & landing after 5 seconds; and the motor will auto-fly to

20 meters if the altitude is less than 20 meters, when aircraft returns the

throttle servo doesnft work, the autopilot will descent to the ground

automatically when reaching the start position and can interferes the landing

position; After enabling gauto-returningh, it canft prevent autopilot from

23

returning and auto-landing if switching CH6 back to gauto-hovering hand

gauto-navigationh, unless switch CH5 to manual position(position1/2) and then

back to holding position to continue hovering.

Notices:

Click on gmotor offh -> gOKh to shut down the motor.

(5)Click & Fly to point

MODE : in GPS auto-hover mode

Step1:

Click any point on the map and it can appear a yellow smile there.

Step2:

Click on gFlyToPPh button in Map page (the button will get gray if there is no click

beyond a few seconds, then need to click on the map again and the yellow smile

will appear ), the yellow round smile will change to purple star smile. The

operation of flying to the next position is same as above.

Remark:

Within 10 meters, craft moves in level ;and beyond 10 meters, the head of craft

faces the target point.

Warning:

If the point clicked on is too far, the craft will fly out of sight, so user need to be

cautious when using the Fly to Point function.

(6)Target Lock

MODE: in auto-hover mode

Solution1:

Step1:

It will appear a yellow smile while clicking any point on the map;

Step2:

24

Click gPTZLockh button in map page. After yellow round smile changing to purple

star smile, the aircraft head will keep facing the locked point;

Solution2:

Step1:

in any time (including before takeoff or manual/auto mode after takeoff)

fly/take the craft to the target point to be the locked , then within 3 seconds

switch between position1 and position3 on CH6 for three times(that is

1->3->1->3->1->3->1 ), FC will record the longitude&latitude of GPS where

the craft locates as the target point to be locked.

Step2:

in GPS auto-hover mode, within 3 seconds switch between position1 and

position2 on CH6 for three times(do not switch to position3 which is invalid, that

is 1->2->1->2->1->2->1 ), then FC will enter target lock mode.

Tips:

After locking the target with solution1 or solution2, users can operate by

transmitter or by RC mode on phone.

Remark1: When left aileron outputs in RC interface, the aircraft will hover

clockwise around the target point. When the right aileron outputs, it will hover

anti-clockwise.

Remark2: Pushing the stick can decrease the hovering radius and drag the stick

can increase the hovering radius.

Remark3: After selecting gQuit FlytoPP/PTZ Lockh in "Toolsh page or switch back

to manual mode, the elevator operation and aileron operation will be normal.

User can switch to manual mode and immediately back to GPS mode to continue

the operation.

i7jWaypoints Flying

Step1:

In Map page of GCS, click gtoolsh-> " Design waypoints" to make the waypoints

25

designing. One click on map once can get one waypoint, get all waypoints one

by one. Click gDefault Toolh to complete the waypoints design.

Tips:

If the firmware version is later than Sept.15th,2012, each waypoint contains

"Height", "Hovering time","Speed" and other key parameters, so must make

sure that click each waypoint with finger to select "edit waypoint" and correctly

setup.

"Height" indicates the point where craft is staying, which can be set as

negative value below zero(when the takeoff position is too high and the craft

need to fly under the takeoff position). If height between two waypoints are

different ,craft will fly as climbing slop. The default value of height is 9999, that

is to say since the altitude is too high to reach, craft will fly according to the

present height.

"Hovering time" means the time that craft stays in the present position before

going to the next waypoint(the default hovering time in first waypoint is 65535

seconds ).

"Speed" means the flying speed that craft flies to the next waypoint from

present position.

Step2:

After the waypoints design is completed, it can be saved as file which can be

used next time. User can also click on "Upload Waypoints" to upload the

waypoints to Autopilot. Please check whether each waypoint has been changed

to blue to confirm a successful upload (orange indicates the upload is not

successful), and then get into "Target Point" to check whether the number

displays there is the same as uploaded waypoints, if not you need to upload

once more;If any waypoint is not changed to blue, please also upload the

waypoints once more.

Step3:

Click gRemove Waypointsh to make the blue waypoints recover to red; then

26

Select gVerify waypointh from gToolh, download uploaded waypoints to ground

Station for comparing, if all waypoints are blue it means the uploaded waypoints

is the same with that in GCS and waypoints verification is successful. Otherwise

need to upload waypoints once more.

Step4:

After switching CH5 to position 3 and CH6 to position 2, then click on "enable

skyway" in setting page of GCS, the craft will fly to the first waypoint and hover

There. Fill in "Change Target" with number 2 and upload it, the craft will fly to

waypoint 2A3A4...and fly back to waypoint 1 and hover there after finishing all

waypoints.

Notice:

If waypoints incorrectly uploaded, when switched to auto-navigation mode the

aircraft will fly far away automatically and out of control.

Free control of craft heading:

In normal condition, when it's waypoints fly, craft nose will face fly orientation in

real time. When user need to freely control the course of craft, in waypoints fly

mode can click on "Intelligent course locked" in Setting page of GCS. At that

time the "Intelligent course locked" doesn't present care free function, it

presents free course mode. That is craft will fly according to "waypoints fly",

however, the craft nose will not face the fly orientation in real time, but the

course of craft can be freely controlled by rudder on RC transmitter. Click on

Disable intelligent course locked and confirm, then the craft nose will face fly

orientation again.

(8) Follow Me

This function must be used in the condition that GPS on phone is enabled and

meanwhile GPS of this phone has located showing on gdatah page of GCS. In

GPS hover mode, after all the sticks on transmitter are set in the neutral position,

user can click on genable followh in gsettinghpage, the craft can fly following the

27

mobile phone GPS and lock the craft nose orientation.

If to quit this function, just click on gDisable followh on gsettingh page.

i9jCare free

In GPS hover mode, "Carefree" can be used.

Option 1

Click on "enable course locked" button in "setting" page, autopilot can record

the current orientation of craft nose. Then users can adjust the orientation of

craft nose with rudder;when users operate the left/right aileron and push/pull

sticks on transmitter, the autopilot will fly to the orientation as the course was

locked, not the current orientation of craft nose.

Option 2

Within 5 seconds please switch between position1 and position2 on CH6 for five

times(do not switch to position3 which is invalid, that is

1->2->1->2->1->2->1->2->1->2->1 ), then FC will enter intelligent course

locked mode.

To exit "carefree", click on "disable course locked" button in "setting" page or

switch to manual mode. User can switch to manual mode and immediately back

to GPS mode to continue the operation.

28

10 Parameter Setup

Default Parameters

The Factory Default Parameters are shown as below. It is the Quad-X flying

mode and can be applied to most crafts for direct flying.

Notice:

hPTZ Roll Sensitivityh & gPTZ Pitch Sensitivityh can directly affect the touch

sensitivity of transmitter.

Please do NOT change the parameters of Roll/Pitch,THR P/I,Rotate P/D. If

parameters are incorrectly changed, can push the THR to the minimum and

switch to the manual mode, then click on Settings .. Default Settings, the

parameters can recover to factory defaults.

The meaning of other parameters are as belowF

Cell Num

The autopilot can calculate the low voltage alert according to the Cell Number

filled in by users. When the mobile vibrates once each 2 seconds, it means the

power is getting low and remind user to note. When the mobile keeps vibrating

it means the power is getting very low and request the craft to land at once.

29

Ctrl Type

Complete default number 2.

1. Attitude mode, fit to adjust parameters/dynamic flight.

2. Acceleration mode, fit to static flight(autostability, common mode).

Magnetic Declination

Fill in local magnetic declination, deflection to West is Positive/to East is

negative ( most regions in China are deflecting to West).

Example: Magnetic declination in Beijing is 6‹ 30f to West, that is 6.5 degree,

then fill in 6.5. When magnetic declination is more than +10degree(to west) or

smaller than -10degree(to east), after that autopilot will rounding down decimal

places and just leave the integers.

Please refer to the below website about magnetic declination

http://magnetic-declination.com/

http://www.ngdc.noaa.gov/geomagmodels/struts/calcDeclinat

Aircraft Type

Fill in mix control type, please refer to Part 6.

PTZ Roll Sensitivity

PTZ Pitch Sensitivity

EXT1 is the PTZ roll output and EXT2 is the PTZ pitch output. Roll/Pitch

sensitivity is used to adjust the compensation angle of PTZ. If user feel the

compensation angle is small, can fill in bigger number, on the contrary fill in

smaller number (Note: negative value can be filled in to be reversed).

When using PTZ, transmitter must be enabled and CH7&CH8 are correctly set to

two knob switches, making it more convenient that autopilot can read the

CH7&CH8 to output the PTZ position.

Attitude Sensitivity

It is the revised coefficient of inclined angle generated by aircraft when resisting

the external force.The bigger the number, the more sensitive to the external

force, default value is 50, fit to most crafts.

30

Max Vertical Speed

In GPS mode, the max default speed of up/down (unit:cm/s) is 200,that is 2m/

sec., the max value can be set as 255.

Motion Compensation

The default value is 80 fit to most crafts. When the sticks back to neutral

position on transmitter, increase the value to get a more stable flying if the

aircraft is shaking heavily. But it will result in high-frequency vibrating if the

number is too big. Setup range:0~255.

Vertical Stable

It is available only in manual mode to increase the comfortable level of THR by

manual, default value 150. The air pressure has big influence on motor stability

when the value is too big. If the air pressure is not stable in windy days, too big

number will affect the comfortable level of THR in manual mode and without

holding altitude.

Max Speed

Setup of maximum flying speed.

Motor arming

Setup of whether to use motor arming to remove throttle protection.

Protection

Setup of whether to enable the THR protection.

Overload control

The acceleration of craft in GPS hover mode can be selected.

Gentle, common and severe are optional.

31

Notice:

The 4 parameters below need to be filled in when use flight controller for first

time, when filling in the values must make sure the THR stick is in the bottom;

click on gSettingsh.. gEnter setting g, when gsetting statush displays in gdatah

interface you can update the parameters. Then click on gsendh to give the

command to autopilot and click on ggeth to confirm the update is correct. After

verifying, click on gSettingsh..gExit settingh, the flying can be started.

Remote Control Mode

User can select the options below according to yourself RC mode, do not set up

incorrectly!

Self-adaptive: AP select the options according to the using RC Tx.The RC Tx

must be enabled first before autopilot enabled.

Normal: Ordinary FUTABA Receiver, AP CH1 connect to Receiver CH1, AP CH2

connect to Receiver CH2.

S-BUS: Only connect AP CH7 to the Receiver S-BUS port.

PPM: PPM is supported, only connect AP CH8 to Receiver PPM.

Volt Alert Threshold

Fill in the alert voltage of each battery which is been using, need user to fill in

32

after measuring by self. Usually fill in 3.65.

ESC Type

Fill in the ESC type which is been using.

Note:

Fault filling of ordinary ESC and XA ESC will result that the propellers go out of

control after power connected, dangerous!

Steering gear Frequency

Input the correct value according to your servo type: 50HZ for Analog Servo,

250HZ and 333HZ for Digital Servo.

11 Power module & Speed sensor

When YS-X6 is connected with power management module, user can directly

observe the present discharge current and battery consumption in gdata h page

of GCS.

If to use this function, user must first connect power management module with

YS-X6(connect to A1N1 plug on YS-X6), push the throttle stick to the bottom,

then repower YS-X6 on. After powering, FC will zero sensor automatically, the

current discharge (ampere A) and battery consumption(MAH) start to output

according to the detection by power management module.

33

12 Data Link connection extension

TipsF

. User can purchase datalink XB-PRO900 on selection to extend the

communication distance, making the GCS and phone control avoid from WIFI

distance limit. For data link XB-PRO900, the transmission power is 100 mW and

frequency 900 MHZ, communication distance is not less than 2 ~ 3 km (open

areas). The physical port of datalink XB-PRO900 is RS232 and the

communication baud rate is 115200 BPS.

. A pair of XB-PRO900 contain two data links, one comes with craft and another

with ground station. But the two encapsulations are same and can be

interchanged.

i1jData link (on Craft) connection

Take off WIFI module from craft and install data link on craft, connect the data

link to YS-X6 COM3 referring to WIFI module connection.

Data link on Craft

i2jData link (on ground) connection

Connect the WIFI module, taken off from the craft, to another data link on

ground (using cables come with data link), which can supply power with

wireless router on ground together. That is there are three parts on the

ground:data link, WIFI module, wireless router.

34

Notice:

. If WIFI module works in point-to-point, there is no need using router.

. Data link and WIFI both are powered on by 3 S ~ 6 S lithium battery (same as

AP MC), the power cable is in red-black two colours, red is positive pole and

black is negative pole.

35

13 Joystick Connection

After purchasing the joystick, users can use it to control the craft in GPS mode.

When data link is not connected, joystick control distance is up to WIFI

communication distance. When using datalinkXB-PRO900, the control distance

of joystick is up to communication distance of datalink .

Joystick contains two parts: joystick and joystick WIFI.

Notice:

The WIFI here coming with joystick is different from the WIFI above in Part12,

they can not be exchanged)

i1jJoystick Connection

The joystick connection is quite simple, just need to connect joystick WIFI to

joystick according to the diagram above, and power it on in a convenient place ,

joystick can work automatically .

Note:

Joystick and WIFI both are powered by 3 S ~ 6 S lithium battery (the same as

AP MC), the power cable is in red-black colour, red is positive pole and black is

negative.pole.

The cable connecting joystick with joystick WIFI is Dupont cable in three colour.

36

i2jJoystick Instructions

1. In GPS auto-hovering mode, joystick can be used to control the craft . After

power on the joystick, joystick operation equals to the cross operation of RC

interface on phone. To avoid conflicting with phone cross operation, please

disable remote control on phone GCS to cancel the round circle on the cross.

2. When RC is off , the round circles on remote control cross interfaceof phone

GCS will display once automatically. At this time the joystick can't be used,

otherwise there will be two cross operations at the same time. If need to use

joystick at that time please select gmenuh and click on g RC offh in phone GCS,

to cancel the round circle on RC cross interface, then the joystick can be used.

37

14 Craft Shaking Adjustment

First, deviation of installation /vibration/shaking will make the test of flying

status by IMU delayed. So user need to check whether the installation direction

of IMU is correct and try to install it on the stable place on craft where there is

no resonance.

Notice:

The IMU of YS-X6 has damper inside, so there is no need using extra damper.

The vibration and shaking coefficient showing on GCS means the shaking status

of IMU. Before allocating motors and propellers, users must make balance test

to reduce the vibration and shaking coefficient smaller than 10, the smaller the

better(try to use the smaller propeller to reduce the shaking). Soft frame will

bring extra shaking, please use frame with better rigidity.

Second, the attitude of Multi-rotor is up to motor speed adjustment, so servo

sensitivity will directly affect the accuracy of Multi-rotor attitude adjustment.

User need to adjust weight and pitch of propeller to be proper, so that motors

can rotate in enough speed and generate enough servo effect. For the craft

seeking for high efficient configuration, it is inevitable that the flight stability will

reduce, user only can select one between flight efficiency and stability;

Third, the symmetry of Multi-rotor has significant effect on flying stability. In

quarter 3 greal servo positionh of Part 9, user can estimate the symmetry of

motor and propeller in level flight. If fail to adjust the symmetry, user must

spend more energy in frame and dynamic configuration;

Fourth, if the three steps above are finished, user can adjust gtoll sensitivityh,

gpitch sensitivityh in g parameters settingh , that is to set the correction strength

of craft; And adjust gmotion compensationh, that is to set the correction of

stabilization of craft to realize a certain kind of adjustment.

Note:

gToll sensitivityh, gPitch sensitivityh have been set the correction value

with same angular velocity error, 60 is default, the bigger the sensitivity the

38

quicker the correction, that is the manual control is more flexible. But too big

sensitivity will lead to high-frequency vibrating, so need to decrease the

sensitivity value for high-speed craft(smaller propellers).

gMotion compensationh is set for the shaking correction of low-speed craft,

80 is default. Without the compensation, since the servo effect of low-speed

craft (bigger propellers)is too small, when sticks on transmitter back to neutral

position, the craft will jerk to be stable. User can gradually increase thegmotion

compensationh parameters to get the best effect without jerking when the sticks

back to neutral position. But for the high-speed craft(smaller propellers), the

value should be decreased, otherwise the craft will highly-frequency vibrate.

The minimum value of the parameter is 0 and the max value is 255.

39

15 Firmware Upgrade

The upgrade of YS-X6 firmware is simple. Before firmware upgrade, please

download the special firmware upgrade tool on Zero UAV official website:

http://www.zerouav.net/kefu.aspx

USB Cable Connection: plug one end of USB cable in box to PC USB port , and

the other end to FC COM1.

Warning:

COM3 is used for connecting to Data link or WIFI, and COM1 for upgrade. Wrong

connection will lead upgrade failure.

Upgrade Steps:

Step1

Please do not power on FC and if FC is powered please disable it.

Enable the special programme "YS-X6 PC Firmware Upgrade" for upgrade ,click

on "AP Firmware Upgrade",the software will be enabled and shown as below:

Step2

Select COM port for upgrade (if do not know which COM on PC, please click with

right mouse gmy computerh->gattributeh->device managerh->

gporth(COM/LPT)to have a check). Click on gConfig COMh, then setup its

property to be(as the diagram below):

Baud rate: 115200

40

Data bite: 8

Parity: None

Stop bit: 1

Data flow control: None

When setting finished, click on gOpen COMh

Step3

Click on Open button, Select a g.armh file offered by Zero UAV to

upgrade(shown as below diagram).

Step4

Repower on the FC and it will start to auto-upgrade. When it says in red words

gPlease Restart AP!h, then autopilot can be powered off.

41

Warning:

1,When all settings are finished, if repower on AP the software doesnft

auto-upgrade, please disable the software and replug the USB cable.

2,If the below warning is showing when you choose firmware versions,

Please redownload the upgrade firmware or update the date later than the

present date(within 30days) , then upgrade once more.

42

Appendix 1: LED Description

Flight States

GPS unlocated, red light blinks for 3 times one loop. œœœ œœœ œœœ

GPS satellites=5, red light blinks for 2 times one loop. œœ œœ œœ œœ

GPS satellites=6, red light blinks for 1 time one loop. œ œ œ œ œ œ

GPS satellites>7, the Led doesn't blink. œ

Abnormal craft status=White light blinks

When GPS from attitude is too large, Led blinks in white(craft status is

abnormal). ...............

When craft gets stable after big action:

If white light is off, craft can continue to fly; œ

If white light keeps going on, need to land craft urgently. ..........

Altitude hold in the Stabilization mode=Blue light periodically blinks

Blue light blinks for one time one loop means users is operating; œ œ œ œ

Blue light blinks for two times one loop means Altitude hold. œœ œœ œœ

GPS mode=Green light periodically blinks

Green light blinks one time one loop means the user is operating; œ œ œ œ

Green light blinks for two time one loop means Auto-hover and Altitude Holding.

œœ œœ œœ œœ œœ œœ

Low voltage alert

Low voltage alert=Red light fast blinks; œœœœœœœœœœœœœ

Low voltage alert urgently=Red light keeps going on. ..........

Compass Calibration

Attitude error< 3‹, light keeps going on, it means can go around to calibrate.

Attitude error> 3‹, light will be off, it means the craft is not placed in horizontal

and need to be adjusted to be level. œ

After Magnetic data calibration, the Purple light keeps going on during data

reading. LED will get normal when data reading is finished. .........

43

Appendix2: Port Description

MC

A/CH1 Aileron-For roll control (left/right)

E/CH2 Elevator-For pitch control (front/back)

T/CH3 Throttle-For throttle control

R/CH4 Rudder-For rudder control

CH5 Manual/Auto Mode Switch

CH6 Auto-Hover/Auto-waypoints/Returning&Landing Switch

CH7 For gimbal pitch servo/ Or for S-BUS

CH8 For gimbal roll servo/ Or for PPM

COM1 Connect RS232 port and to PC for firmware upgrades.

COM2 Extended port

COM3 Connect WIFI /Or Data link

AI0

AI1 Connect Power Calculate Module

LED Connect LED Indicator

P To AP power supply

IMU1 To power

IMU2 To data

M1 Motor 1

M2 Motor 2

M3 Motor 3

M4 Motor 4

M5 Motor 5

M6 Motor 6

M7 Motor 7

M8 Motor 8

EXT1 Gimbal roll servo

EXT2 Gimbal pitch servo

EXT3 Extended/ Or for Camera

GPS1 To GPS power

GPS2 To GPS data

44

Appendix 3: GCS Interface Introduction

The software interfaceF

Buttons and Functions

No. Buttons Functions

1 Remote Control Default interface (Cross interface) to control the aircraft

2 Data Real time flight data

3 Map Real time map or saved map

4 Settings Setup various states of craft

5 Parameters Adjust craft parameters

6 Cross Interface General control the flight

7 The Cross A track to control the craft to Climb up/ Go down/Rotate

8 Round Circle in

Cross

A button to control the flight by phone

9 Enable/Disable

Attitude Control

Used to Enter/Exit phone attitude control mode

10 Quit Quit from the GCS

11 Enable/Disable

Remote Control

Used to Enter/Exit phone control mode

12 Network Used to set service information

13 Enable Bluetooth Used to enable bluetooth

45

Note:

When the data is connected, the Serial No. and firmware version will display on

the remote control interface.

Data

Get into the data interface by touching the button "Data", shown as below:

It will display "Data disconnect" on the interface if data is not connected.

It shows how many satellites were researched and reached.

GPS Velx(cm)

GPS Vely(cm):

GPS velocity. It displays a value below 10 when the aircraft is static.

46

Xekf Velx

Xekf Vely

Xekf Veld:

The velocity after Kalman filting. It displays a value between 0-20 and increase

constantly. When the velocity exceeds normal range, LED will be in solid white,

suggest you land the aircraft urgently.

Display real time attitude duiring flight.

Display real attitude of the static aircraft. Static Angle can work only with no any

vibration, used to check whether the aircraft is in all good conditions before

takeoff.

You can fill the waypoints number which you want the aircraft fly to.

It displays the craft states: Manual, Manual Altitude Hold, Auto Hover, Auto

Navigation, Auto Go Home and Land, setup status.

Barometer altitude of the craft, unit is meter.

Course Angle means the directions which the crafthead points to. Due North=0

degree, clockwise direction is positive, counterclockwise direction is negative.

For example: Due East=+90 degree, Due West=-90 degree, Due South=+180

degree or -180 degree.

Notice: It works only when in GPS mode and flying in Auto Modes, it will not

work when in Manual mode or Attitude mode.

Autopilot voltage

47

GPS longitude and latitude.

Target point longitude

Target point latitude

Longitude and latitude of target point.

Display of rudder, aileron, elevator, throttle and manual servo position. Please

check more details in "RC Transmitter Settings".

Vibrations from IMU.

1-10 means your aircraft is in good condition.

10 above , it means vibration exceeds limits.

Up&down is the vertical vibrate and front/back/left/right is the horizon vibrate

of IMU,

Display of shaking state

1-10 means your aircraft is in good condition.

48

Map

+: Zoom in

-:Zoom out

Map: map mode.

Satellite: satellite mode

Tips:

1 The map which you used last time was saved in default, when you open it next

time, it shows up the saved map.

2 There is no network. Enable WIFI, cached data can be displayed.

Cached map: when network is on, enable the software to download the map. If

finishing downloading cached map, enable WIFI, the cached map will be

displayed.

Locate

Locate Plane :

After the GPS was located, click this button can find where your aircraft locates.

My Location:

Phone/Tablet position. The GPS of Phone/Tablet need to be enabled.

Search Location: (Search the position where you want to check)

Save Location : (Save the current position on map)

49

Load Location: (Load the saved position on map)

Fly ToPP

Fly to Point Fly. Please check "Fly To Point Fly" page.

PTZ Lock:

Target Lock Fly. Please check "Target Lock Fly" page.

Tool

Click it to enter editing waypoints, please check " Waypoints Fly" Page.

Locate FlytoPP/PTZLock: Move Map center to operation area.

Exit FlytoPP/PTZLock: Remove the FlytoPP/PTZLock mode.

Waypoint edition

Click on waypoint, there will be a text box display as below:

50

Single waypoint edit

Used to edit waypoint attribute, such as longitude and latitude, altitude and

speed.

Single waypoint upload

To upload the waypoint.

Single waypoint delete

To delete the waypoint

Number

Waypoint number.

Altitude

The height of waypoint, unit: meter

Longitude

Longitude of this waypoint.

51

Latitude

Latitude of this waypoint.

Hovering time

Time of hovering, unit is second, zero means no hover(the default hovering

time is 65533 seconds of first waypoint).

Camera setting

Optional of camera attribute.

Target speed

It means the flying speed of craft from present position to next waypoint.

Settings

Notice:

All operations in settings are vitally important, please operate seriously.

Click it enter into "Magnetic Compass" Calibration.

It's used to capture Neutral Position after flight and Transmitter adjustments,

please operate it by manually in no wind.

52

Notes: When capture neutral position, please move all sticks to middle position.

It has nothing to do with THR position.

It's used to change home position or record new home position.

Auto Go Home and Landing. After the aircraft reaches home location, the craft

can be controlled manually.

Take off fully automatically, please check Part 9 (3) gAuto Take Off".

Enable receiver/ Disable receiver

It is strongly suggested for new operator that do not disable receiver when

flying.

This function can be used only when your phone has GPS hardware and got

more than 5 satellites locked, and in Auto-hover mode. After enabled Follow Me,

the aircraft will fly automatically following mobile GPS location.

Note: Generally, phone position has error about 5~10 meters.

In Auto Navigation mode, fill in the new waypoint number which

you will fly to.

Attention:

Fill in a number which is in the range of uploaded waypoints number.

53

In Auto-Hover mode, it's used to set flight altitude, put in altitude needed here.

Example: filling in 10 means setting flight altitude as 10 meters.

Please confirm to align the channel without any electric power connected.

Example: When moving Throttle stick as Minimum-Maximum-Minimum, if GCS

data displays as below: Manual Throttle is 7 when Throttle stick is at the

Minimum position and 90 when at the Maximum position, that means Channel

Align is completed successfully.

Special Attention

Please don't configure propeller when power first time, only power the autopilot

and need to calibrate Channel Align first.

Operation as blow:

Select "Channel Align" in Settings, push/pull Elevator sticks of RC Transmitter to

the Maximum/Minimum, move Aileron sticks of RC Transmitter to the

leftmost/rightmost within 5 seconds. After 5 seconds, autopilot will complete

collecting data automatically. User can observe whether the data shown in GCS

can match with real manual operation when was moving the sticks.

Example: When pulling Throttle stick to the Minimum, the " Throttle" showing

in GCS displays 7; Pushing to the Maximum, it displays 90.

It's used to align channels or complete parameters of craft. After clicking this

button, flight state will display "Settings". After getting into settings, the control

channels for each motor on craft is directly connecting to throttle channel of RC

Transmitter and no any mix control output, it is equal to like a Y-line of "one to

many" , it's convenient for users to set strikes for all rotors at the same time.

After completing Throttle channel calibration and all parameters filled, click this

button to exit from settings.

Enable Skyway/Disable Skyway

The craft can be switched to Auto-navigation mode only after set CH6 to

Auto-navigation and then click this button "Enable Skyway" to open the

waypoints first.

Enable Skyway/Disable Skyway is used when receiver is off.

54

Click on "Function Limit", the dialog box will show as below:

click "Get"

Users can observe the functions opened in yourself FC.

User can define the parameters by yourself.

55

Mix-control

The up/down,turning, pitch ,toll of multi-rotor is caused by mix-add of different

output of motors, and the control process is called mix-control. The ratio of

mix-output is up to the mechanical structure of craft. User can configurate the

output coefficient of motors to make the fix-control realized. Please refer to the

Part 6 Craft Type for details.

Click on "install guide". User can install the software following the guide step by

step.

Intelligent course lock

To enable the intellignet course lock. It can be used in GPS auto-hover mode.

Exit intelligent course lock

To quit intelligent course lock.

56

Appendix4 Configurate WIFI with N to WIFI without N

Warning

Please read the instructions below carefully and severely modify the

configuration step by step.

1 Login the webpage

When WIFI module, whose serial No. comes with "N",is powered on and works,

a AP with name of YS-X6-Serial No.N (Example:YS-X6-12345.N) will be builded.

Users can connect net AP with PC, and type in password:12345. Then open the

browser and type in the IP Add.192.168.1.254, the webpage will shown as

below, then type in the user name and password (Username:admin

password:admin),click Login to enter the module allocation page.

2AChange DHCP Client to Disable

Click on gBasichpage to find DHCP Client, and change it to Disable, shown as

below:

57

After modification, please click on Apply, and wait for 20seconds, so the IP

address of WIFI will be locked as192.168.1.254. Then refresh the browser and

relogin the module allocation page.

3AClick on WIFI interface

Please change Network Mode to Infrastructure mode,Channel to AUTO, Service

Area Name/SSID is the same as router SSID, the Security Mode to AES mode.

Click on Apply after changing, and reboot 20 seconds later.

4DRouter connection

Connect PC to the integrated router, example: if YS-X6-12345 router is

connected, type in password 12345678.

Type in 192.168.1.254 in the browser,it indicates the modification is successful

if you can login the back interface of WIFI. After power on the AP, user can

enable the phone GCS, after 5 seconds the GCS can receive the data of flight

controller.

Note: Do not change the data which is not referred to above.

58