YS-X6 Multi-Rotor Autopilot

User Manual

V1.2

www.zerouav.com

Warning and Declaimer

- The manual contains information about installation, debugging，and how to use the product. Please read it thoroughly before using the product.

- Zero UAV (Beijing) Intelligence Technology Co. Ltd. assumes no liability for damage(s) or injuries incurred directly or indirectly from the use of this product.

- Please keep far away from the crowd, children and property when using the product.

- Install propeller after completing all the debugging and checking, to avoid damage(s) or injuries.

5. When any of the following events or incidents has taken placed, we will not offer any warranty and service:

- The product has been repaired, modified, or any parts of the product have been substituted or replaced by anyone not expressly authorized by Zero UAV.

- The warranty card, the serial number of the hardware and the flight data or any of these items is lost.

- Damaged caused by use's faults such as attempting wiring not in accordance with the manual, which may cause short circuit.

- The user manual would be modified when the firmware and specifications are changed. Please visit the official website of Zero UAV http://www.zerouav.com for the updates, or contact directly the factory, and authorized distributors.

Contents

Safety Instructions…………………………………………………………………………………

YS-X6 Features……………………………………………………………………………………

In Box………………………………………………………………………………………………

Quick Start Guide…………………………………………………………………………………

Assembly…………………………………………………………………………………………

GC Software……………………………………………………………………………………

GC software Installation…………………………………………………………………

Firmware Upgrade………………………………………………………………………………

Flight Data…………………………………………………………………………………

GC Software Interface …………………………………………………………………

Remote Control…………………………………………………………………………

Data……………………………………………………………………………………

Map……………………………………………………………………………………

Setup…………………………………………………………………………………………

Parameter…………………………………………………………………………………

Test Fly…………………………………………………………………………………………

Fly………………………………………………………………………………………………

Magnetic compass Calibration  …………………………………………………

Waypoints Flight……………………………………………………………………………

Brief Troubleshooting …………………………………………………………………

Appendix

Port Definition……………………………………………………………

supported Multi-Rotor……………………………………………………

LED Description………………………………………………………………………….

Safety Instructions

For safety reason, please adhere to the below procedures:

- Please install propeller only after everything has been properly, and ensure that the setup and installation has been completed.

- The side marked "IMU" of the IMU faces up. The arrow points to the front of the aircraft.

- Make sure power on the RC transmitter first then power on YS-X6 autopilot before takeoff; Power off  YS-X6 autopilot first, then power off  RC transmitter after landing. Pay serious attention when using the S-BUS or self-adaptive feature of the autopilot.

- Throttle calibration, manual servo position, realtime servo position, channel settings must be accurate.

- GPS+COMPASS is sensitive to magnetic interference, should be far away from any electronic devices. The arrowhead points to head of aircraft while installing..

- Please set F/S function of RC transmitter before the flight.

- Do not fly in GPS mode when the signal is not good (red light blinks) or the static GPS speed is more than 15cm/s

- The Gimbal Servo can be supplied power from output of any ESC, but when ESC has no BEC output, please add extra power not use the power supplied from Autopilot.

- When returning home in Auto-returning home Mode, the two ways can shut down motors: When THR of RC Transmitter is maintained at the range of 0-10%; When staying at the bottom of cross operation interface(click “ Control” in GCS). These two above ways can not work to shut down motors when in other Flight Mode.

- The low voltage protection function is NOT for experimenting, please care about your flight time and try best to avoid using low voltage protection.

- Please don’t launch your craft if the GCS displays very high “vibrate state”.

- Must check whether the “course angle” and” magnetic declin” data are correct after compass magnetic calibration.

- Must check whether “Attitude angle” and“Static angle” are maintaining consistency before takeoff.

- When connecting the wires of GPS+COMPASS、IMU and MC, please pay attention to keep the color of the connectors and the labeling on the Unit body consistent. Wrong connection may result in device short-circuit.

- Prevent Anti-plug Design The mechanisms to protect from short circuitry is NOT for fun, please don’t try.

- Please don’t fly in GPS mode when course angle and real rotational angle loss consistency in some strong magnetic field, and recalibrate the compass magnetic.

- The system is in default when RC Receiver is on and it is in auto-flight mode when RC Receiver is off. Please note the state of RC Transmitter when powering on the RC Receiver.

YS-X6 Features Introduction

YS-Series

YS-X6 is an excellent Multi-Rotor Heli autopilot, it supports common third-party commercial ESC and quad-rotor / Hex-rotor/ octo-rotor with common custom mixed control. YS-X6 offers GPS Stabilization and Auto-hover functions from quad-rotor to octo-rotor.

In Box

■Hardware □Software（Need Download）

■Main Controller(MC) X1

The MC combines with other modules, communicates with external electronic devices to carry out autopilot functionality. Update firmware combining with PC R232 COM port. YS-GCS software record real time flight state via WIFI

■IMU X1 (Inertial measurement unit)

Contains sensors to measure the attitude of the craft.

■GPS+COMPASS X1

The GPS/Compass module is for sensing the position and direction.

■LED Indicator X1

The LED indicates current flight states of the craft via light.

■WIFI X1 （The Data radio can be added to spread range based on this）

1. Connect it via Mobile/Tablet hotspot function. 2. Connect AP to Ground Station via WIFI.

■GPS Bracket X1 GPS＋COMPASS is sensitive to magnetic interference, please use this bracket to mount GPS module when necessary and keep it far away from EMI sources.

■3-PIN Servo Cable X8 Used to connect MC to RC Receiver.

■R232 COM port to 3-PIN Servo Cable X1 Used to connect PC to MC, Program Upgrade only.

■3M Gummed Paper (Double-side adhesive tape) Used to fix components on multi-rotor frame

■Warranty Information Card X1 It provides Product Serial No., Purchase Date. Please fill out related information and return back to Zero UAV for registering your product warranty.

□GS Software for Android System □GS Software for Apple System □Firmware Upgrade Software on PC.

Quick Installing Guide

Step 1:

Configure Hardware reference to “Assembly”, Please check Page XX

Step 2:

Install Software reference to “GS Software Installation”, Please check Page XX.

Step 3:

Setup RC Transmitter, please check Page XX.

 FUTABA RC Transmitter needs to make a fresh new model memory. Reverse channels for other RC Transmitters when necessary. Example: JR. WFLY need to reverse all channels and calibrate frequency.

 Set CH5 & CH6 to two 3-position switch on RC Transmitter.

Step 4:

Power YS-X6, connecting autopilot to GCS through WIFI, then

check whether GCS Interface displays fresh data which are keeping walking.

Step 5:

Calibrate Channels reference to “Channels Calibration”, please check Page XX.

Step 6:

Fill in parameters, please check PageXX

Step 7:

Set ESC Stroke (endpoint).

Ways: Get into the “Settings” on GCS without any power connected, push THR stick to the maximum position and then power the craft, after twice sound “di” push THR stick to the bottom position, meanwhile ESC will report the cell number of battery (one “di” sound means one battery). Exit from the “Settings” after completing setup.

Step 8:

Check the mixing mode, with propeller taken out for safety reason.

Ways:

Stand at the tail of the craft and watch the front. Move the Aileron stick to the left and the left motor should stop running, do the same to the right motor. Push the ELE stick to the top and the front motor stop running, pull back ELE stick to the bottom and the rear motor stop running.

Step 9:

Calibrate the compass (request to perform this in flying field), Please check Page XX

Step10:

Test fly, please check Page XX

Notice：

★ Except particular notice on wires connection, the black one of all wires should be connected to the bottom of the pin.

★While connecting, all wirings with color should match the same color like showing as below diagram.

★Connection labels are facing you.

★The numbers of the connectors match those marked in the diagram.

1. GPS module plug (Black)                 16. CH1

2. GPS module plug (Red)                   17. CH2

3. Camera                                   18. CH3

4. Gimbal Tilt (Pitch)                         19. CH4

5. Gimbal PAN (Roll)                         20. CH5

6. Motor8                                    21. CH6

7. Motor7                                    22. CH7 (shared with S.BUS)

8. Motor6                                    23. CH8

9. Motor5                                   24. Link to PC (Firmware Upgrade)

10. Motor4                                   25. NOT USED

11. Motor3                                   26. WIFI Module output

12. Motor2                                   27. Power voltage (above 4S)

13. Motor1                                   28. Current sensor

14. IMU module plug (Black)                  29. LED indicator

15. IMU module plug (Red)                   30. Battery (Power)

RC Transmitter Settings

Please read this introduction thoroughly. Because all flying mode is based on using Channel 5 and Channel 6 of the RC Transmitter. Both channels must be set to 3-position switches which is for switching Manual/Auto/Altitude hold 3 modes. Users can check the working state of the switches in the “Flight Mode” on GCS Interface. Please familiarize with the following switch position:

|  | CH5 |  |  | CH6 |
| --- | --- | --- | --- | --- |
| Position 1 | Manual | Red light blinks for 3 times |  | × |
| Position 2 | Stabilization Mode and Altitude Hold | Blue light blinks twice |  | × |
| Position 3 | GPS Mode | Green light blinks twice |  | 1.Auto-hovering 2.Auto Navigation/waypoint 3. Auto-returning home |
| Remark1 | 1. After switching to GPS Mode, CH6 is available. 2. Disengage any flight mode in GPS mode, need to disengage the GPS mode. Example: If you want to fly in Auto navigation/waypoint mode, set CH5 to GPS mode first, then set CH6 to Auto navigation mode. If need to disengage the Auto navigation/waypoint mode, please set CH5 to Manual mode or Stabilization/Altitude Hold mode. |  |  |  |
| Remark2 | 1. “Auto-returning home” for standard version: altitude can be controlled after reaching returning home position. After landing the craft manually, the motors can be shut down only upon switching to stabilization mode. 2. “Auto-returning home” for Enterprise version: aircraft can land automatically when reaching returning position meanwhile 11 %-90% THR can not work, 0-10% THR makes the motors power off immediately. |  |  |  |

CH5 position

Position 1. Manual Mode: all flight characteristics can be controlled manually.

Position 2. Stabilization Mode (Manual) and realize altitude hold through barometer sensor. User can control the aircraft manually but altitude is controlled by the autopilot. We recommend to set CH5 to position 2 before takeoff  (the motors may roll slightly) and then push ELE stick to launch the craft; In position 2, the middle THR position is for altitude hold, push THR stick up to make the craft climb up, push THR stick down to make the craft go down.

Position 3. GPS Mode (Auto Mode), please check the instruction of CH6 position showing as blow to activate the sub modes in GPS Mode. Also can operate this way: after capturing the middle position of the craft before takeoff, set CH5 to position 3 and CH6 to position 1, then push THR stick and the craft can take off  automatically immediately.

CH6 Position( Notice: it can work only with setting CH5 to position 3 first)

Position 1. Auto-hovering in GPS Mode, the craft will hold its position with releasing the sticks. Pushing/Pulling Elevator stick or moving the Aileron stick left/right to make the craft move. After releasing the sticks, the craft will recover to auto-hovering state.

Position 2. Auto-navigation/waypoint. The craft will fly away without uploading the waypoints before switching to this position, So please make sure the waypoints have been uploaded successfully before switching to this position.(Please check Page XX for waypoint design and upload)

Position 3.  Auto-returning home. For setting returning home position, please check Page XX (Returning home position).

When any accident unexpected happen, please switch back to manual mode, in case the craft flies away.

Attention: Set F/S of RC Transmitter as Auto-returning home, throttle should be kept at the range of 10% above even more, to avoid motor power off when throttle range is reaching 10% or less. Recommend users to set 30% as the throttle range when returning home and landing.

Ground Control Software (GCS)

Installation

Step1: Download YS-GCS, installing it according to operation system tips.

YS-GCS can be installed easily for Android. Installation can be completed only need to download the GCS to mobile and run it once in file manager.

Apple installation please check Page XX

Step 2: Connection between Autopilot and GCS

Way1: Connect Autopilot to GCS through WIFI Router (Recommendation)

Way2: Build Hotspot through Mobile/Tablet.

As WIFI Module was set to default search and connect wireless network named “YS-X6 Serial no.” example:YS-X6-10200 and password is 82890430. MUST use WPA2-PSK AES encryption method. After the above wireless network was built successfully, open WLAN and connect it to the router by mobile/tablet, the communication can be established with Autopilot & Router powered on, meanwhile realtime data can be displayed in Mobile GCS. (Namely data in GCS keep walking means WIFI was connected successfully.)

When using Hotspot by Mobile, it can be connected only with IP Address 192.168.1.X. , otherwise please add a router to connect.

For example: If Mobile IP is:192.168.0.1,the Hotpoint function can not work.

Default User Name:YS-X6-Serial No., password: 82890430

Firmware Upgrade

Step1: Autopilot can not be powered. Connect 3 pin connector to the COM1 port of the autopilot and 9 pin connector (DB9) to PC COM port.

Step2: Run firmware upgrade program (download from Zero UAV official website), then select “configure COM port” on PC.

In the device manager of operation system, you can see which COM port is being used, then set its property to be:

Baud rate: 115200

Data bite: 8

Parity: None

Stop bit: 1

Step3: Select a “.arm” file offered by Zero UAV to upgrade.

Step4: Power the autopilot, Upgrade can be completed automatically. When it says in red words “Upgraded, please close the window.” , can shut up the autopilot.

Flight data

All the realtime flight data will be saved automatically to a “hj” folder under the local file named “T+runtime” in mobile when flying. The flight tracking file for Apple is saved to the folder:/private/var/mobile/documents/yswifi

When users meet a flight problem, can send it back to the factory if need. This file is also one of the necessary documents for warranty, please preserve carefully.

GCS Interface

Software Functions:

| No. | Content | Functions |
| --- | --- | --- |
| 1 | Control | Default interface. Flight interface by Mobile |
| 2 | Data | Realtime flight data of the craft |
| 3 | Map | Saved map or realtime map |
| 4 | Settings | Set craft states |
| 5 | Parameters | Adjust parameters of the craft |
| 6 | Control Interface | Mix Control by Mobile |
| 7 | Center Button | Control the aircraft by mobile |
| 8 | Control Cross | Control the craft to Climb/ Go down/rotate by mobile |
| 9 | RC on/off | Control the craft by mobile after opening it. |
| 10 | Magnetic field data | Show the Magnetic field calibration results |
| 11 | IP Searching | Reserved |
| 12 | Quit | Quit from the software |

Control

After switching the RC Transmitter to Auto-hovering mode in GPS mode, click mobile menu, if mobile has got into the Control then you can use the Mobile to control the craft.

Operation Listed as below:

1.The craft will Auto-hover with altitude hold and position hold when no touching the GCS control cross.

2.Press down the middle circle and keep it, moving your finger to left/right/front/back or to any direction, the craft will move same like when using the aileron and elevator sticks of RC Transmitter. （In order to classify the difference from adjusting flight direction/ altitude , mobile will vibrate once when start to operate.)

3. When moving your figure on the green cross (no pressing the middle circle), up/down is for adjusting altitude, the more higher position means the faster climbing speed, the more lower position means the lower declining speed. The craft will hold its position and altitude when releasing your finger. Left/right is for for changing the flight direction, the same operation as up/down.

4.When the craft flies out of WIFI communication range and the autopilot can not  receive mobile signal, it will be automatically switched to accept signal from RC Transmitter. When the craft flies back to the WIFI range, mobile can be automatically recovered to control the craft again.

The Remote control functions shown as below:

| "Control" open | Operation method on "Control" interface | Aircraft states |
| --- | --- | --- |
| Circle area | Red: unlocated; Green: located. | Keep still |
| Hover Operation | Do NOT touch any area | Altitude hold and Position hold. |
| Level flying Operation | - Press hold the center circle and move up/down - Press hold the center circle and move left/right - Press hold the center circle and move left top, left bottom, right top, right bottom. | - move to front or back - Move to left or right - Move to left top/left bottom/right top/right bottom. |
| Climb/ Go down/Rotate Operation | - Skip touching any position to right top from the circle(Within the green cross area) - Skip touching any position to right bottom from the circle(Within the green cross area) - Skip touching any position to left from the circle(Within the green cross area) - Skip touching any position to right from the circle(Within the green cross area) Attention: - The distance to the red dot is the flight speed. The further the faster. - Press holding time is flight time. - The craft will auto-hover when releasing your finger. - The Maximum speed is 2.5 m/s | 1. Climb 2. Go down 3. Rotate to the left 4. Rotate to the right |
| The bottom of the green cross | Activate "Disable motors” only when in auto-returning home, please use carefully. | Shut down the rotors immediately |
| WIFI data lost when flying | The craft will auto-hover with exceeding WIFI range and switch to be controlled by RC Transmitter. | Auto-hover |
| Reget WIFI signal | Automatically Start " Enable control" | Depend on current mobile states |

Data

Get into the data interface by touching the button "Data", shown as below:

GPS Satellites:

Locate GPS Satellites Quantity.

GPS Velx(cm)

GPS Vely(cm):

GPS speed. Position hold is better when the value is below 10.

Xekf Velx

Xekf Vely

Xekf Veld:

The speed after Kalman filting. When the flight speed exceed normal range, the LED of autopilot will indicate solid white, means obstacle happened and need to land urgently. Value below 20 is normal when the craft is static on ground.

Attitude Angle:

Real time attitude when flying.

Static Angle:

Real Attitude data when the craft is static. Static Angle can work only with no any vibration. When existing different value with checking on ground, need to operate " Zero Gyro" in GCS.

Target Point:

Waypoints number, upload the waypoints number to the craft which will fly, it can not exceed the total waypoints or keep as 0.

Flight Mode:

Craft states: Zero Gyro, Manual, Auto-waypoints, Auto-navigation, Return home and land, settings.

Flight Altitude:

Barometer altitude of the craft, unit is meter.

Course Angle:

Course Angle means the directions which the craft head points to. Due North=0 degree, clockwise direction is positive, counterclockwise direction is negative. For example: Due East=+90 degree, Due West=-90 degree, Due South=+180 degree or -180 degree.

Note: The Course Angle can mean real flight direction only with GPS was located and switched to Auto Mode.   It has no physical meaning in other circumstances.

AP Volt:

Autopilot voltage ( has loaded)

GPS Lat

GPS Lng

GPS longitude and latitude.

Manual Servo Position

Rudder

Elevator

Aileron

Throttle

For example: When RC Transmitter is in the center position and throttle is the minimum, it displays rudder:“ Middle", throttle:7,  if deviation value is more than 2, need to capture center position.

Vibrate State:

It displays the influence of vibration from different directions on IMU. Shock absorption effect is not good when vibration value is more than 10.

For example: up/down 15 means the vibration influence on IMU from up/down direction is relatively great.

Map

+:Zoom in

-:Zoom out

Map: map mode.

Satellite:

satellite mode (select according to the real situations)

Tips:

- The map will stay back to the last one by default when get   into it next time.

- When network is not connected, click to open WIFI, it will display cache data.

Locate Map

Locate Plane : (Craft position)

Click this button to find the craft position after GPS located.

My Location:  (My position)

Search Location: (Search Position)

Save Location (Save Position)

Load Location (Load position)

Fly ToPP

It only can be used in Auto-hovering state. Click any position on the map and a yellow smile will show up there, then click the button ‘Fly ToPP“ the yellow smile shall change to purple smile,after that the craft will fly to the position and hover there.